# Albion University Network Design — Documentation

Coursework project: design and implement a two-campus university network in Cisco Packet Tracer, using VLANs, router-on-a-stick inter-VLAN routing, router-based DHCP, RIPv2 for internal routing, and static routing to an externally hosted email server.

## 1. Project overview

Albion University has two campuses, 20 miles apart.

Main campus

- Building A — Admin staff (Management, HR, Finance) + Faculty of Business. Devices get IP addresses automatically from a router-based DHCP server.

- Building B — Faculty of Engineering/Computing + Faculty of Art/Design.

- Building C — Student labs + IT department (hosts the university's web server and other internal servers).

Smaller campus

- Faculty of Health and Sciences — staff and student labs on separate floors.

External

- An email server hosted on the cloud, reached over a static route (not part of the internal routing protocol).

Every department/faculty sits on its own IP subnet. RIPv2 handles routing between all internal routers; a static route handles reachability to the external email server.

## 2. Assumed IP addressing plan

The brief doesn't give exact addresses, so this plan uses a simple, expandable scheme. Swap in real numbers if your assignment specifies them — the design logic stays the same.

| Location | VLAN | Department | Network |
| --- | --- | --- | --- |
| Building A | 10 | Management | 192.168.1.0/24 |
| Building A | 20 | HR | 192.168.2.0/24 |
| Building A | 30 | Finance | 192.168.3.0/24 |
| Building A | 40 | Faculty of Business | 192.168.4.0/24 |
| Building B | 50 | Engineering/Computing | 192.168.5.0/24 |
| Building B | 60 | Art/Design | 192.168.6.0/24 |
| Building C | 70 | Student Labs | 192.168.7.0/24 |
| Building C | 80 | IT Department (servers) | 192.168.8.0/24 |
| Smaller campus | 90 | Health & Sciences — Student Labs | 192.168.9.0/24 |
| Smaller campus | 100 | Health & Sciences — Staff | 192.168.100/24 |

| WAN link | Network | Purpose |
| --- | --- | --- |
| Main Campus Router ↔ Branch Campus Router | 10.10.10.4/30 | Internal — carried by RIPv2 |
| Main Campus Router ↔ Cloud Router | 10.10.10.0/30 | External — static route only |
| Email server (cloud) | 20.0.0.0/30 | Reached via the static route above |

## 3. Devices used

- 2911 routers — Main Campus Router, Branch Campus Router, and a "Cloud" router representing the external network hosting the email server.

- 3560 multilayer switch — aggregates Buildings A, B, and C on the main campus.

- L2 switches — one per department, connected up to the multilayer switch.

- End devices — one PC and one printer per department, plus the internal servers in Building C.

## 4. Step-by-step implementation

### Step 1 — Design the topology

Lay out the routers, multilayer switch, L2 switches, and end devices in Packet Tracer before configuring anything. Getting the diagram right first makes every later step faster, because you already know which interface connects to what.

### Step 2 — Connect routers with serial DCE cables

Router-to-router links (Main↔Branch, Main↔Cloud) use serial DCE cables. Insert the DCE serial module into each router first (Physical tab, power off, drag the module in, power back on) — 2911 routers don't have serial ports built in by default.

Why: serial links simulate the long-distance WAN connection between campuses, and to the cloud-hosted email server, since a normal copper Ethernet cable isn't realistic over that kind of distance.

If skipped: there's no way to model the WAN connection at all — the two campuses (and the email server) would be physically unreachable from each other in the topology.

### Step 3 — Wire everything else

Connect switches to routers, and end devices to switches, with copper straight-through cables.

### Step 4 — Turn on all interfaces

enable

configure terminal

interface <interface or range>

no shutdown

Why: every interface on a router or switch starts in a shutdown (administratively down) state by default. Nothing passes traffic until it's turned on.

If skipped: the interface stays down no matter how correctly everything else is configured — pings, DHCP, routing, all of it fails on that link, and it can look like a much bigger problem than it actually is.

### Step 5 — Set the clock rate on serial DCE interfaces

enable

configure terminal

interface <serial interface>

clock rate 64000

do write

Why: a serial link needs a timing signal so both ends agree on bit boundaries. On a real line this normally comes from an external device; in Packet Tracer, the DCE end of the cable has to generate it. Only the DCE end needs this command — check the cable icon (a small clock symbol) to confirm which end that is.

If skipped: the serial interface will typically show up/down instead of up/up, and no traffic will pass across that link — meaning the Main↔Branch and Main↔Cloud connections won't work at all.

### Step 6 — Create VLANs on the L2 switches

enable

configure terminal

interface range fa0/1-24

switchport mode access

switchport access vlan 10

do write

Why: each department needs to be its own separate broadcast domain and IP network, exactly as the brief requires. VLANs are what let one physical switch serve multiple isolated departments.

If skipped: every device plugged into that switch would sit in the same default VLAN (VLAN 1) — no separation between departments, no way to give each one its own IP subnet, and no way to apply different security policies per department.

### Step 7 — Create VLANs and a trunk on the multilayer (L3) switch

! Port facing an L2 switch — access port for that department's VLAN

interface fa0/1

switchport mode access

switchport access vlan 10

do write

! Port facing the router — trunk, carries every VLAN

interface gig1/0/1

switchport trunk encapsulation dot1Q

switchport mode trunk

do write

Why: the L3 switch sits between the department L2 switches and the router. Ports facing individual L2 switches only need to carry that one department's VLAN, so they're access ports. But the single link going up to the router has to carry all the VLANs from Buildings A, B, and C at once — that's only possible on a trunk, which tags each frame with its VLAN ID as it crosses the link.

If skipped (access instead of trunk on the router-facing port): only one VLAN's traffic would ever reach the router. Every other department would have no path to their gateway, no internet access, and no way to reach IT's servers or the email server.

### Step 8 — Assign IP addresses to router and switch interfaces

enable

configure terminal

interface se0/0/0

ip address <ip> <subnet mask>

do write

Why: every routed interface (serial WAN links, router-to-switch links) needs an IP address before it can send or receive routed traffic, or be advertised by a routing protocol.

If skipped: the interface has no identity on the network — RIPv2 has nothing to advertise for it, and no device can use it as a next hop.

### Step 9 — Inter-VLAN routing (subinterfaces) + DHCP

Subinterfaces:

enable

configure terminal

interface gig0/0/0.10

encapsulation dot1Q 10

ip address <first host IP> <subnet mask>

do write

Why: one physical cable runs from Building A's switch up to its router. A single physical interface can only ever belong to one IP subnet — but Building A alone has four separate VLANs (Management, HR, Finance, Business), each needing its own router-based gateway. A subinterface is a logical split of that one physical port, one per VLAN, each with its own IP address, each recognizing only the traffic tagged for its VLAN.

If skipped: only the one VLAN matching the physical interface's own IP would work. The other three departments in Building A would have no gateway — no communication outside their own VLAN, and (since DHCP for a VLAN is normally tied to its subinterface) no dynamic IP addressing either. This directly breaks the requirement that Building A devices get their address dynamically from a router-based DHCP server, and breaks the requirement that every department is reachable end-to-end.

DHCP:

configure terminal

service dhcp

ip dhcp pool <department-pool>

network <network address> <subnet mask>

default-router <first host IP of that network>

dns-server <dns ip>

do write

Why: the brief specifically requires Building A devices to get addresses dynamically. service dhcp turns the feature on; each ip dhcp pool defines one department's scope, with default-router telling clients their gateway (which must match that VLAN's subinterface IP).

If skipped: Building A PCs get no IP address automatically — they'd need manual static configuration on every device, which fails the assignment's explicit requirement, and doesn't scale as more devices are added.

### Step 10 — Enable RIPv2

enable

configure terminal

router rip

version 2

network <network address>

do write

List every directly connected network on that router (VLAN subinterfaces + WAN links).

Why: a router only automatically knows about networks it's directly connected to. Without a routing protocol, Building A's router has no idea Building C's subnets even exist, and can't forward traffic to them. RIPv2 automates the exchange of that reachability information between all the internal routers, so every router learns about every other router's networks without you typing static routes by hand everywhere.

If skipped: devices could talk within their own department, but nothing would be able to reach another department, another building, or the other campus — directly breaking the "end-to-end connectivity" requirement in the brief.

Note on the external email server: the brief specifically asks for static routing to the external server, not RIPv2. That's because the email server sits outside the university's own network (hosted externally on the cloud) — you don't control or trust that external network's routing the same way, so a manually configured static route is the appropriate, deliberate way to reach it, rather than dynamically learning routes to/from an outside network.

! On the Main Campus Router, pointing to the cloud-hosted email server network

ip route 172.16.100.0 255.255.255.0 <next-hop IP toward Cloud router>

### Step 11 — Verify

show ip interface brief     ! confirm interfaces/subinterfaces are up/up

show vlan brief              ! confirm VLANs and port assignments

show interfaces trunk        ! confirm the L3-to-router link is trunking

show ip route                ! confirm RIP-learned routes (R flag) and the static route (S flag) to the email server

show ip dhcp binding          ! confirm Building A devices are receiving leases

ping                          ! test reachability between departments, campuses, and the email server

### Step 12 — Done

Once every department can reach every other department, the branch campus, and the external email server, the core connectivity requirement of the brief is met.

## 5. Quick reference: why each piece matters

| Component | Why it's needed | What breaks without it |
| --- | --- | --- |
| VLANs | Keep each department on its own separate network, as required | No department isolation; can't give each one its own subnet |
| Trunk (L3 switch → router) | Carries every department's VLAN over one physical link | Only one VLAN would ever reach the router |
| Subinterfaces | Let one physical router port serve multiple VLANs, each with its own gateway | Three out of four Building A departments lose their gateway entirely |
| Router-based DHCP | Required specifically for Building A by the brief | Devices get no automatic IP address |
| RIPv2 | Lets internal routers learn about each other's networks automatically | No department can reach another building or campus |
| Static route (email server) | Deliberate, controlled path to an external, untrusted network | No reachability to the email server at all |

Documented as part of Albion University's network design coursework — Task 1–3 (Cisco Packet Tracer topology, configuration, and evaluation report).